DIVA ENGINEERING Leek PCB Keychain Hardware Design Document ¦ Magical Mirai Concert Swag

| Document ID | HW-LEEK-001 |
| --- | --- |
| Version | 1.0 |
| Date | March 2026 |
| Author | Diva Engineering |
| GitHub | https://github.com/diva-eng |
| Status | Draft — Ready for PCB Design |

# 1. Project Overview

The Leek PCB Keychain is a small-form-factor PCB designed to be distributed as concert swag at Magical Mirai events. Shaped like Hatsune Miku's iconic leek prop, it features green LEDs at the tip and white LEDs at the base, driven by a microcontroller with multiple user-selectable lighting patterns. The device is powered by a coin cell battery and includes a power switch and a pattern selection button.

## 1.1 Design Goals

- Leek-shaped PCB outline, keychain form factor

- Green LEDs at the tip (leek top), white LEDs at the base (cut end)

- 4 stored lighting patterns, selectable via a tactile button

- SPDT slide switch for hard power on/off

- CR1220 coin cell power — small enough for keychain use

- Fully assembled by JLCPCB (SMT + THT)

- Firmware written in Arduino IDE using megaTinyCore

## 1.2 Project References

- Related project: BOKAKA smart PCB card (https://github.com/diva-eng/BOKAKA)

- Related project: diva-card RP2040 dev board (https://github.com/diva-eng/diva-card)

- Org GitHub: https://github.com/diva-eng

# 2. Hardware Specification

## 2.1 Microcontroller — ATtiny402

The ATtiny402 in SOT-23-8 is selected as the MCU. It is a Microchip AVR 0-series device with sufficient I/O for this application, native PWM for LED fading, and extremely low sleep current. It is supported by JLCPCB assembly and programmable via UPDI using Arduino IDE with megaTinyCore.

| Parameter | Value |
| --- | --- |
| Package | SOT-23-8 |
| Flash | 4KB |
| SRAM | 256 bytes |
| EEPROM | 128 bytes |
| Operating Voltage | 1.8V – 5.5V |
| Active Current @ 1MHz | ~0.8mA |
| Active Current @ 32kHz | ~100µA |
| Power-down Sleep | ~0.1µA |
| PWM channels | 2 (TCA0) |
| Programming Interface | UPDI (single wire) |
| LCSC Part | C440337 |

## 2.2 Pin Allocation

| Physical Pin | Port | Arduino Pin (megaTinyCore) | Function |
| --- | --- | --- | --- |
| 1 | VCC | — | Power supply |
| 2 | PA6 | 1 | Green LED group output |
| 3 | PA7 | 2 | White LED group output |
| 4 | PA1 | 4 | UPDI programming |
| 5 | PA2 | 3 | Pattern select button input |
| 6 | PA3 | — | Spare / future use |
| 7 | GND | — | Ground |
| 8 | PA0 | — | RESET (not used) |

## 2.3 Power — CR1220 Coin Cell

The CR1220 is chosen as the primary power source. It offers a good balance of small physical size and sufficient capacity for concert use. Since the device includes a hard power switch, battery life in active use is the only relevant metric.

| Parameter | Value |
| --- | --- |
| Chemistry | Lithium manganese dioxide |
| Diameter | 12mm |
| Thickness | 2.0mm |
| Nominal Voltage | 3V |
| Capacity | ~35mAh |
| Estimated active runtime | ~16 hours @ 2.1mA total draw |
| Concerts covered | 5+ at 3 hours each |
| LCSC Holder Part | C70377 |

## 2.4 LED Specification

Green LEDs (Leek Tip — 4x)

| Parameter | Value |
| --- | --- |
| Color | Green |
| Package | 0402 SMD |
| Forward Voltage (Vf) | ~2.1 – 2.3V |
| Target current per LED | 2mA |
| Series resistor | 390Ω |
| LCSC Part | C2297 (verify Vf in datasheet) |
| Qty | 4 |

White LEDs (Leek Base — 2x)

White LEDs have a forward voltage near the coin cell supply voltage. Use low-Vf white LEDs (Vf ≤ 2.8V) to ensure reliable operation.

| Parameter | Value |
| --- | --- |
| Color | White (low Vf) |
| Package | 0402 SMD |
| Forward Voltage (Vf) | ≤ 2.8V (verify before ordering) |
| Target current per LED | 2mA |
| Series resistor | 68Ω |
| LCSC Part | C2290 (verify Vf ≤ 2.8V in datasheet) |
| Qty | 2 |

## 2.5 Passive Components

| Ref | Component | Value | Package | LCSC Part | Qty |
| --- | --- | --- | --- | --- | --- |
| C1 | Bypass capacitor | 100nF 10V | 0402 | C307331 | 1 |
| R1–R4 | Green LED resistors | 390Ω | 0402 | C105872 | 4 |
| R5–R6 | White LED resistors | 68Ω | 0402 | C105854 | 2 |

## 2.6 Switches

| Ref | Component | Type | Package | LCSC Part | Qty |
| --- | --- | --- | --- | --- | --- |
| SW1 | Power switch | SPDT slide | SMD | C431540 | 1 |
| SW2 | Pattern select | Tactile push button | 3x4mm SMD | C455280 | 1 |

# 3. Full Bill of Materials

All parts are sourced from LCSC and suitable for JLCPCB SMT + THT assembly. Verify all LCSC part numbers and datasheets before placing the order, especially LED Vf values.

| Ref | Component | Value / Part | Package | LCSC Part | Qty | Assembly |
| --- | --- | --- | --- | --- | --- | --- |
| U1 | MCU | ATtiny402 | SOT-23-8 | C440337 | 1 | SMT |
| C1 | Bypass Cap | 100nF 10V | 0402 | C307331 | 1 | SMT |
| R1–R4 | Resistor | 390Ω | 0402 | C105872 | 4 | SMT |
| R5–R6 | Resistor | 68Ω | 0402 | C105854 | 2 | SMT |
| D1–D4 | Green LED | Low Vf Green | 0402 | C2297 | 4 | SMT |
| D5–D6 | White LED | Low Vf White ≤2.8V | 0402 | C2290 | 2 | SMT |
| SW1 | Power Switch | SPDT Slide | SMD | C431540 | 1 | SMT |
| SW2 | Pattern Button | Tactile 3x4mm | SMD | C455280 | 1 | SMT |
| BT1 | Battery Holder | CR1220 | THT | C70377 | 1 | THT |
| J1 | UPDI Header | 3-pin 2.54mm | THT | C49257 | 1 | THT |

Note: J1 (UPDI header) can be replaced with 3 exposed edge pads for pogo-pin programming after the first batch is validated, removing the need for a physical header.

# 4. Circuit Description

## 4.1 Power Path

The CR1220 positive terminal connects through SW1 (SPDT slide switch) to the VCC rail. The negative terminal connects directly to GND. C1 (100nF ceramic) is placed as close as possible to the ATtiny402 VCC pin to decouple supply noise.

CR1220(+) ── SW1 (slide) ── VCC rail ── ATtiny402 VCC

CR1220(-) ──────────────── GND

C1 (100nF) between VCC and GND, placed near U1

## 4.2 LED Wiring

Each LED has an individual series resistor. All LEDs in a group share the same MCU output pin. Cathodes connect to GND.

ATtiny402 PA6 ──┬── R1 (390Ω) ── D1 (Green) ── GND

├── R2 (390Ω) ── D2 (Green) ── GND

├── R3 (390Ω) ── D3 (Green) ── GND

└── R4 (390Ω) ── D4 (Green) ── GND

ATtiny402 PA7 ──┬── R5 (68Ω)  ── D5 (White) ── GND

└── R6 (68Ω)  ── D6 (White) ── GND

## 4.3 Pattern Select Button

SW2 connects PA2 to GND. The ATtiny402 internal pullup is enabled in firmware, so no external pullup resistor is required. Button press pulls PA2 low, triggering a pattern change with software debounce.

ATtiny402 PA2 ── SW2 ── GND

(Internal pullup enabled in firmware — no external resistor needed)

## 4.4 UPDI Programming Header

J1 is a 3-pin 2.54mm THT header exposing GND, VCC, and UPDI (PA1) for in-system programming. Place on the board edge for pogo-pin access.

J1 Pin 1: GND

J1 Pin 2: VCC

J1 Pin 3: UPDI (PA1)

# 5. LED Patterns

Four patterns are stored in firmware. The active pattern is saved to EEPROM so it persists across power cycles. Patterns are cycled by pressing SW2 (short press).

| # | Name | Green (PA6) | White (PA7) | Description |
| --- | --- | --- | --- | --- |
| 1 | Slow Pulse | PWM breathe | PWM breathe | Both LEDs fade in and out together slowly |
| 2 | Alternate | Flash on | Flash off (inverse) | Green and white take turns flashing |
| 3 | Chase | On → Off → Both | Off → On → Both | Sequential: green, white, both, off |
| 4 | Heartbeat | Double-flash + pause | Double-flash + pause | Quick double-flash then a long pause |

PWM fading uses the ATtiny402 hardware TCA0 peripheral. No bit-banging is required. The 32kHz internal oscillator is sufficient for smooth visible fading.

# 6. Firmware

## 6.1 Toolchain

| Tool | Version / Details |
| --- | --- |
| IDE | Arduino IDE 2.x |
| Board Package | megaTinyCore by Spence Konde |
| Board Manager URL | http://drazzy.com/package_drazzy.com_index.json |
| Board Selection | ATtiny402 |
| Clock | Internal 32kHz (sufficient for LED PWM) |
| Programmer | SerialUPDI or Microchip SNAP |

## 6.2 Firmware Structure

#include <Arduino.h>

#include <EEPROM.h>

#define PIN_GREEN   PIN_PA6

#define PIN_WHITE   PIN_PA7

#define PIN_BUTTON  PIN_PA2

#define NUM_PATTERNS 4

uint8_t currentPattern = 0;

void setup() {

pinMode(PIN_GREEN, OUTPUT);

pinMode(PIN_WHITE, OUTPUT);

pinMode(PIN_BUTTON, INPUT_PULLUP);

currentPattern = EEPROM.read(0) % NUM_PATTERNS;

}

void loop() {

if (digitalRead(PIN_BUTTON) == LOW) {

delay(50); // debounce

if (digitalRead(PIN_BUTTON) == LOW) {

currentPattern = (currentPattern + 1) % NUM_PATTERNS;

EEPROM.update(0, currentPattern);

while (digitalRead(PIN_BUTTON) == LOW);

}

}

switch (currentPattern) {

case 0: patternSlowPulse(); break;

case 1: patternAlternate(); break;

case 2: patternChase(); break;

case 3: patternHeartbeat(); break;

}

}

## 6.3 EEPROM Usage

The last active pattern index is stored at EEPROM address 0 using EEPROM.update() (only writes if value changed, preserving EEPROM write cycle life). On boot, this value is read and restored.

# 7. Programming Setup

## 7.1 SerialUPDI Programmer (Recommended)

A CH340 or CP2102 USB-to-serial adapter with a 1kΩ resistor bodge is sufficient for programming. This is the lowest-cost option and fully supported by megaTinyCore.

USB-Serial TX ── 1kΩ ── UPDI pin (J1 Pin 3)

USB-Serial RX ──────── UPDI pin (J1 Pin 3)

GND ────────────────── GND         (J1 Pin 1)

(Power the board from the coin cell during programming)

## 7.2 Microchip SNAP (Alternative)

The Microchip SNAP (~$35 USD) is the official programmer and supports UPDI natively. Recommended if this project leads to further ATtiny development at Diva Engineering.

## 7.3 Batch Programming Jig

For production batches, expose 3 pads on the PCB edge (GND, VCC, UPDI) and use a 3D-printed pogo-pin jig to program boards without soldering headers. Each board programs in approximately 5 seconds.

| Pad | Signal | Notes |
| --- | --- | --- |
| J1 Pin 1 | GND | Connect to programmer GND |
| J1 Pin 2 | VCC | Monitor only — power from coin cell |
| J1 Pin 3 | UPDI (PA1) | Single-wire programming data |

# 8. PCB Design Notes

## 8.1 Form Factor

- Leek-shaped outline — elongated with a bulbous bottom (white LEDs) tapering to a tip (green LEDs)

- Keychain hole: 3mm diameter, reinforced with annular copper ring, placed at the base

- Target max dimensions: ~30mm length × 10mm width

- PCB thickness: 1.6mm standard

- Silkscreen: Include Diva Engineering logo and Magical Mirai year on back

## 8.2 Component Placement

- U1 (ATtiny402): Center of board, away from LEDs to avoid thermal interference

- C1 (100nF bypass): As close as possible to U1 VCC pin

- D1–D4 (Green LEDs): Along the upper PCB outline (leek tip region)

- D5–D6 (White LEDs): Along the lower PCB outline (base region)

- SW1 (Slide switch): Bottom edge, accessible with thumb

- SW2 (Tactile button): Front face, easy to press

- BT1 (CR1220 holder): Back of PCB, centered

- J1 (UPDI header): PCB edge, or replace with 3 exposed pads for pogo programming

## 8.3 Design Rules for JLCPCB

| Parameter | Value |
| --- | --- |
| Min trace width | 0.127mm (5mil) |
| Min clearance | 0.127mm (5mil) |
| Min drill size | 0.3mm |
| Copper layers | 2 (F.Cu, B.Cu) |
| Surface finish | HASL or ENIG (ENIG recommended for 0402) |
| Solder mask | Green (or custom color — white for leek aesthetic) |
| Silkscreen | White (F.Cu side) |

## 8.4 JLCPCB Assembly Notes

- All SMT components (U1, C1, R1–R6, D1–D6, SW1, SW2) eligible for Economic PCBA

- THT components (BT1, J1) eligible for THT assembly — include in same order

- U1 (ATtiny402) ships blank from JLCPCB — firmware must be flashed after delivery

- Mark component orientation clearly on silkscreen, especially D1–D6 LED polarity

- Confirm CR1220 holder footprint matches C70377 before submitting Gerbers

# 9. Power Budget

The device is switched off between uses. Battery life is calculated for active (on) use only.

| Component | Current Draw | Notes |
| --- | --- | --- |
| ATtiny402 @ 32kHz | ~0.1mA | Running pattern logic |
| Green LEDs × 4 @ 2mA each | ~8mA peak | Dependent on PWM duty cycle |
| White LEDs × 2 @ 2mA each | ~4mA peak | Dependent on PWM duty cycle |
| Total (all on, peak) | ~12.1mA | Worst case, all LEDs at full brightness |
| Total (typical active) | ~2–4mA | Average with PWM patterns at ~25% duty |

| Scenario | Current | CR1220 (35mAh) Life |
| --- | --- | --- |
| Peak (all LEDs fully on) | ~12.1mA | ~2.9 hours |
| Typical (PWM patterns) | ~2–4mA | ~9–17 hours |
| Conservative (dim + slow) | ~1mA | ~35 hours |

A 3-hour concert is well within range under any realistic usage pattern. Upgrading to CR1632 (120mAh, 16mm diameter) would provide 3–4× the runtime with minimal size increase.

# 10. Open Items & Pre-Order Checklist

| # | Item | Status | Owner |
| --- | --- | --- | --- |
| 1 | Verify white LED Vf ≤ 2.8V on LCSC C2290 datasheet | Open | Hardware |
| 2 | Verify green LED Vf ~2.1–2.3V on LCSC C2297 datasheet | Open | Hardware |
| 3 | Confirm CR1220 holder C70377 footprint dimensions | Open | Hardware |
| 4 | Confirm slide switch C431540 footprint | Open | Hardware |
| 5 | Design PCB outline (leek shape) in KiCad | Open | Hardware |
| 6 | Confirm JLCPCB THT assembly availability for BT1 | Open | Manufacturing |
| 7 | Write and test all 4 LED pattern functions in firmware | Open | Firmware |
| 8 | Test EEPROM pattern persistence across power cycles | Open | Firmware |
| 9 | Build SerialUPDI programmer dongle (CH340 + 1kΩ) | Open | Tools |
| 10 | Order 3–5 prototype boards before full batch | Recommended | Project |

# 11. Revision History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.0 | March 2026 | Diva Engineering | Initial design document |